Product Features	Applications

- 3100~3500MHz	• Military Radar

- 42.3dBm Output Power @ 14dBm, 3300MHz	• Commercial Radar

- 64% Power-added Efficiency @ 42.3dBm

- Large Signal Gain 28dB @ Pin=14dBm

- Internally 50ohm Matched

- GaN on SiC Technology

Description

Package Type : PP-3G

The PRM33016-4S is a fully integrated GaN Hybrid power amplifier module designed for applications in S-band military and commercial Radar Systems covering 3100~3500MHz. The PRM33016-4S provides 28dB of large-signal gain and 64% power added efficiency while achieving over 16W of saturated output power. The PRM33016-4S is matched to 50 ohms with integrated DC blocking capacitors on both I/O ports. This device's size is 8x14x2.6mm and is packaged in a ceramic surface mount package.

Lead-free and RoHS compliant.

Typical Performance	(VDS= +24V, TC=25℃, 50Ω)

|  | Parameter*1 |  |  |  |
| --- | --- | --- | --- | --- |
| Frequency [MHz] | Saturated Power [W] | Output Power [dBm] @ Pin = 14dBm | Power Added Efficiency [%] @ Pin = 14dBm | Large Signal Gain @Pin =14dBm |
| 3100 | 16.4 | 42.1 | 64.7 | 28.1 |
| 3300 | 17.8 | 42.4 | 66.2 | 28.4 |
| 3500 | 17.5 | 42.4 | 64.8 | 28.4 |

Note

*1 Measured in the PRM33016-4S test board amplifier circuit, under Pulsed CW @ Pulse Width 200 usec, Pulse Period 1 msec

Absolute Maximum Ratings

| Rating | Symbol | Value | Unit | Condition |
| --- | --- | --- | --- | --- |
| Drain to Source Voltage | VDSS | 100 | V | Tc=25℃ |
| Gate to Source Voltage | VGS | -10, +2 | V | Tc=25℃ |
| Operating Voltage | VDD | 52 | VDC |  |
| Storage Temperature | TSTG | -40, +125 | ℃ |  |
| Case Operating Temperature | TC | -35, +85 | ℃ |  |
| Operating Junction Temperature*1 | TJ | 225 | ℃ |  |
| Soldering Temperature*2 | TS | 250 | ℃ |  |

Note

*1 Continuous use at maximum temperature will affect MTTF.

*2 Refer to the Application Note(AN-002) on soldering - "Solder Condition for RFHIC's GaN Device"

C

Electrical Characteristics*1 (T =25℃ unless otherwise noted)

| Characteristics | Conditions |  | Symbol | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC Characteristics (Main) |  |  |  |  |  |  |  |
| Maximum Forward Gate Current | Tc= | 25℃ | IGMAX | - | - | 5 | mA |
| Maximum Drain Current*2 | Tc= | 25℃ | IDMAX | - | - | 3.2 | A |
| Power Dissipation | Tc= | 85℃ | PDMAX | - | - | 29.8 | W |
| Gate Threshold Voltage | VDS = | 10V | VGS(TH) | -3.80 | -3.00 | -2.30 | VDC |
|  | ID = | 5.2mA |  |  |  |  |  |
| Gate Quiescent Voltage | VDS = | 24V | VGS(Q) | -3.75 | -2.80 | -2.25 | VDC |
|  | ID = | 80mA |  |  |  |  |  |
| Drain-Source Breakdown Voltage | VGS = | -8V | VBR | 150 | - | - | V |
|  | ID = | 5.2mA |  |  |  |  |  |
| Saturated Drain Current*3 | VDS = | 6V | IDS | 4.2 | 5.2 | - | A |
|  | VGS = | 2V |  |  |  |  |  |
| Gate Leakage Current | VGS = | -8V | IGLKG150 | -1.6 | - | - | mA |
|  | VDS = | 150V |  |  |  |  |  |
| Drain Leakage Current | VGS = | -8V | IDLKG150 | - | - | 2.1 | mA |
|  | VDS = | 150V |  |  |  |  |  |
| DC Characteristics (Drive) |  |  |  |  |  |  |  |
| Maximum Forward Gate Current | Tc= | 25℃ | IGMAX | - | - | 1 | mA |
| Maximum Drain Current*2 | Tc= | 25℃ | IDMAX | - | - | 0.8 | A |
| Power Dissipation | Tc= | 85℃ | PDMAX | - | - | 8 | W |
| Gate Threshold Voltage | VDS = | 10V | VGS(TH) | -3.80 | -3.00 | -2.30 | VDC |
|  | ID = | 1.2mA |  |  |  |  |  |
| Gate Quiescent Voltage | VDS = | 24V | VGS(Q) | -3.75 | -2.80 | -2.25 | VDC |
|  | ID = | 40mA |  |  |  |  |  |
| Drain-Source Breakdown Voltage | VGS = | -8V | VBR | 150 | - | - | V |
|  | ID = | 1.2mA |  |  |  |  |  |
| Saturated Drain Current*3 | VDS = | 6V | IDS | 0.8 | 1.1 | - | A |
|  | VGS = | 2V |  |  |  |  |  |
| Gate Leakage Current | VGS = | -8V | IGLKG150 | -0.4 | - | - | mA |
|  | VDS = | 150V |  |  |  |  |  |
| Drain Leakage Current | VGS = | -8V | IDLKG150 | - | - | 0.5 | mA |
|  | VDS = | 150V |  |  |  |  |  |

Note

*1 Measured on wafer prior to packaging.

*2 Current Limit for long term, reliable operation

*3 Scaled from PCM data.

C

Electrical Characteristics*1 (T =25℃ unless otherwise noted) - continued

| Characteristics | Conditions | Symbol | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
|  | RF Characteristics | (Operating Frequency Range) |  |  |  |  |
| Output Power @ Pin=14dBm*1, 2 | Frequency = 3.1GHz | POUT |  | 42.1 |  | dBm |
|  | Frequency = 3.3GHz |  |  | 42.4 |  |  |
|  | Frequency = 3.5GHz |  |  | 42.4 |  |  |
| Power Added Efficiency @ Pin=14dBm*1, 2 | Frequency = 3.1GHz | η |  | 64.7 |  | % |
|  | Frequency = 3.3GHz |  |  | 66.2 |  |  |
|  | Frequency = 3.5GHz |  |  | 64.8 |  |  |
| Large Signal Gain*1, 2 | Frequency = 3.1GHz | GP |  | 28.1 |  | dB |
|  | Frequency = 3.3GHz |  |  | 28.4 |  |  |
|  | Frequency = 3.5GHz |  |  | 28.4 |  |  |
| Duty Cycle | Operating Freq. Range | DC |  | 20 | 30 | % |
| Pulse Width | Operating Freq. Range | PW |  | 200 | 500 | μs |
| Amplitude Pulse Droop | Operating Freq. Range | Droop |  | 0.3 | 0.5 | dB |
| Rise Time | Operating Freq. Range | tr |  | 20 | 100 | ns |
| Fall Time | Operating Freq. Range | tf |  | 20 | 100 | ns |

Note

*1 Pulse width 200μsec, Pulse period 1msec.

*2 Measured in the PRM33016-4S test board amplifier circuit, biasing Drive : ΔVgs-d = -0.5V at Idq 40mA, Main Idq : 80mA

* Above electrical specifications are measured by connecting electrolytic condenser 470μF to VDS. Please make sure that electrolytic condenser is connected properly while testing the PAM.

Block Diagram

Test Board Component Layout with Coin Embedded

PAM

| Part | Description | Part Number | Manufacturer |
| --- | --- | --- | --- |
| C4, C6, C14, C15 | 1.0uF / 100V | GRM21BC72A105KE01 | MURATA |
| R1, R2 | 0ohm, 1608, 0.1W, 50V, -55~155°C | MCR03 EZP0000 | ROHM |
| PCB | 2Layer, 20mil, 1oz | RO4350B | ROGERS |
| CON1~2 | 2.54mm Male Connector | 5267-04A | MOLEX |
| SMA1~2 | Female Connector | - | - |
| PAM | GaN Hybrid PAM | PRM33016-5S | RFHIC |

Package Dimensions	(Type:PP-3G)

- Unit: mm[inch] | Tolerance: ±0.15[.006]

| Pin Description |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 4 | 5 | 10 | 13 | 14 | 15 | 16 |
| RF In | - | - | RF Out | VDS | VGS-m | VDS | VGS-d |
| - | - | - | - | Main | Main | Drive | Drive |

- Other Pins are GND.

Recommended Footprint & Coin Embedded

Package Outline

3.4

[.134]

1.8

[.071]

Via Hole O   0.3

[.01]

2

[.079]

0.8

[.031]

Coin Embedded 11.6x 5.6 / R1.0

ANSI/ESDA/JEDEC Standard JS-002

C2

CDM

ANSI/ESDA/JEDEC Standard JS-001

1C

HBM

Standard

ESD Class

ESD Sensitivity Ratings

600 pcs on a 13" reel (Standard)

PRM33016-4S

Description

Part Number

Packing Information

Lot Number Description

| 2 | 5 | 1 | 4 | 0 | 1 |
| --- | --- | --- | --- | --- | --- |
| ① | ② | ③ | ④ | ⑤ | ⑥ |

①②	Year Number

③④	Week Number

⑤⑥	Production Number

Revision History

| Part Number | Release Date | Version | Description | Data Sheet Status |
| --- | --- | --- | --- | --- |
| PRM33016-4S | Sep, 2026 | 0.1 | Initial Release | Preliminary |

Certification

This product is manufactured by a company that is certified for the AS9100D quality management system.

RFHIC Corporation reserves the right to make changes to any products herein or to discontinue any product at any time without notice. While product specifications have been thoroughly examined for reliability, RFHIC Corporation strongly recommends buyers to verify that the information they are using is accurate before ordering. RFHIC Corporation does not assume any liability for the suitability of its products for any particular purpose, and disclaims any and all liability, including without limitation consequential or incidental damages. RFHIC products are not intended for use in life support equipment or application where malfunction of the product can be expected to result in personal injury or death. Buyer uses or sells such products for any such unintended or unauthorized application, buyer shall indemnify, protect, and hold RFHIC Corporation and its directors, officers, stockholders, employees, representatives and distributors harmless against any and all claims arising out of such unauthorized use. All sales inquiries and support should be directed to the local authorized geographic distributor for RFHIC Corporation. For customers in the US, please contact the US sales team through our website at https://rfhic.com/rfhic-us/. For all other inquiries, please contact our international sales team through our website portal at https://rfhic.com/contact/